ELECTRON DIFFRACTION PATTERN PROGRAMS

-------------------------------------

1.  BACKAUTO:

------------

PROGRAM TO CALCULATE THE MEAN RADIAL DENSITY (BACKGROUND)

OF AN ELECTRON DIFFRACTION PATTERN - any size rectangular area.

The new version has options to carry out automatic centre

determination, and update of the header into the invisible IEXTRA

records used by the subsequent program PICKYCOR.

VX2.3     RH      16-Aug-1993     change highest density from 1024 to 32000

Plots three possible background raster choices for PICKYCOR

- only if IEXTRA entries in header are set for DX1,DY1,DX2,DY2

Plots radial background curve

Plots X-average, Y-average, and perimeter Y-average (YCOR)

Data input cards :

0.	IREF,IPLOT	if IREF.eq.'F' no automatic centring

if IPLOT.eq.F' no plots done.

1.	CX,CY		Centre coords relative to centre of film

2.	NPLATE		Film number used to identify outputs

3.	TITLE		Title for same purpose

4.	NPNTS		smoothing over 2*NPNTS+1 in radial curve

5.	IRMAX,IRMIN	max and min limits for X,Y-average plots

6.	IRMAXC,IRMINC	max and min limits for YCORRECTION

2.  AUTOINDEX:

-------------

This program reads in a digitized electron diffraction pattern, to which

background correction has already been applied, searches for peaks,

averages the image areas centred on each peak, then performs a radial

spiral search until the two shortest base vectors are found.

These are the A- and B-axes.

In subroutine REINDEX, which can be a dummy subroutine, the base vector

indexing can be rearranged to fit with a crystal-specific scheme defined

by the user.   For example, purple membrane hk requires I(4,3) >> I(3,4).

The lattice parameters together with suggested integration and

background raster sizes are put into the header records of the input file

and the output average picture file.

VX1.0	RH	22-Mar-1992	original program

VX1.1	RH	 8-Apr-1992	various debugs

VX1.2	RH	13-Apr-1992	more debugs

VX1.3	RH	19-May-1992	CX,CY debug

VX1.4	RH	21-Mar-1993	default NSTEP=4

VX1.5	RH      16-Aug-1993     always return from SEARCH on failure

Output, for cosmetic examination only, is average picture near an average

spot, together with the assigned indexing, on OUT

There is also a plot file showing the selected rasters on FOR008

For input, requires only the file name of the background-corrected

pattern, on IN

3.  PICKAUTO:

------------

VX3.00	25.3.92		RH

VX3.01	08.4.92		RH	radii in Angstroms, rather than pixels.

VX3.02	22.5.92		RH	resolution statistics

VX3.03  21.3.93         RH      default NSTEP=4

derived from PICKYCOR  much modified program for e.d. spot integration.

NOW DOES A YCORRECTION TO REMOVE DENSITOMETER OD DRIFT (JMB) APPROX 1984.

WITH ADDITIONS TO DEAL AUTOMATICALLY W. TLTAXA,TLTANG 25.6.82.

READS RADIAL AVERAGE BACKGROUND CURVE WITH STANDARD DEVIATIONS

TESTS POINTS IN BACKGROUND RASTERS AGAINST MEAN AND STNDEV AT

APPROPRIATE RADIUS; REPLACES VALUE BY MEAN VALUE AT THIS RADIUS

IF DEVIATION IS MORE THAN 3*STNDEV; KEEPS COUNT OF NUMBER OF

SUCH POINTS -- MAY 82 .JMB.

NOW PLOTS RESIDUAL SPOT C.OF G. VECTOR ON UNIT 8, 14.1.81

OFFSET R**3 DISTORTION CORRECTION AND AUTOMATIC MAXRAD CUTOFF 14.1.81

B3 NOW SEARCHED WITHOUT OFFSET -MAY 82- MORE STABLE ALGORITHM.

PICKTILT WITH RADIAL BACKGROUND CORRECTION, 19.5.80

PICKOFF, JOYCE-LOEBL AND NIKON VERSION 29.11.79

CORRECTS FOR CURVED EWALD SPHERE ROUGHLY, 20.1.81

PROGRAM TO INTEGRATE SPOTS ON A LATTICE, STARTING FROM A RECTANGULAR

RASTER SCAN OF THE AREA . EG . OUTPUT FROM ROYAL OBSERVATORY SCANNER

DATA CARDS ARE

1.  A,B,G,ANGDIS,SHRINK,TILTDIR,KV,LPRINT

REAL SPACE CELL DIMENSIONS AND GAMMA ANGLE, ANGLE ON FILM

PERPENDICULAR TO WHICH A SHRINKAGE BY FACTOR SHRINK IS TO BE

APPLIED BEFORE CALCULATING FINAL TLTAXIS AND TILTANGLE.

TILTDIR - DIRECTION OF TILT ON MICROSCOPE,

-1 = CLOCKWISE

+1 = COUNTERCLOCKWISE

LPRINT  - T=PRINT ALL REFLECTION INFORMATION

2.  J,K

LOOK-UP TABLE FOR OPTICAL DENSITY; LOOKUP(J)=K

THIS IS LINEARLY INTERPOLATED FOR J=1,1500.  THE FIRST CARD MUST

HAVE J=1 AND THE LAST J=1500.

3.  TITLE FOR LOOK-UP TABLE

4.  NPLATE

PLATE NUMBER

5.  TITLE

TITLE OF DIFFRACTION PATTERN, FOR USE ON OUTPUT FILE.

6.  B1, B3

RADIAL DISTORTION PARAMETERS, ONLY B3 USED CURRENTLY.

7.  X0,Y0,

7a. TLTAXA,TLTANG

COORDINATES OF CENTRE, POSITION OF TILTAXIS (ANGLE FROM TILTAXIS

TO A-AXIS IN DIRECTION A TO B POSITIVE), SIZE OF TILTANGLE,

TLTAXA,TLTANG OVERRIDE VALUES CALCULATED INTERNALLY, UNLESS = 0.

NOTE-----TLTAXA HERE IS ANGLE BETWEEN TILTAXIS AND ASTAR ON FILM.

8.  DX1,DY1,DX2,DY2

POSITION OF (1,0) AND (0,1) RELATIVE TO CENTRE. IF ZERO, THESE

ARE TAKEN FROM INPUT FILE HEADER.

9.  ROUT,RIN,PRPMAX

OUTER AND INNER RADII ; MAXIMUM PERPENDICULAR DISTANCE FROM

TILT AXIS FOR SPOTS TO BE INTEGRATED

10. NXM,NYM,NSM,NXB,NYB,NSB,NPOS (TWICE),NTYPE

NXMT,NYMT, ETC.

PEAK AND BACKGROUND RASTERS AND POSITION OF BACKGROUND.

THESE NUMBERS PUT IN TWICE IF IT IS A TILTED DIFFRACTION

PATTERN, TO GIVE RASTER SIZE, ETC AT MOST BLURRED PART OF PATTERN.

NPOS=1 IS AT CENTROID OF TRIANGLE, NPOS=2 IS TWICE AS NEAR SPOTS, ETC.

NTYPE=0,OR 1 DETERMINES POSITION OF 6 BCKGROUNDS AS SHOWN IN

OUTPUT OF BCKGROUND PROGRAM

NTYPE=2 GIVES 4 BCKGROUNDS AT CENTRES OF LATTICE;

11.  FRACT,ABSOL,XAMINE,NCYC,NCYC1

REFLECTIONS WITH FRIEDEL DIFFERENCES GREATER THAN FRACT AND ABSOL

ARE REJECTED.  WEAK REFLECTIONS ARE REJECTED FROM CENTRE OF GRAV.

AND LATTICE PARAMETER CALCULATIONS BY A CRITERION USING XAMINE.

NCYC=N FOR FIXED NUMBER OF CYCLES=N

NCYC=0 MEANS GO STRAIGHT TO DATA ASSESSMENT USING INPUT PARAMETERS

NCYC1=1 FOR PRINT ALL CENTERING RASTERS ON FIRST CYCLE

12.  NELIM

IF +VE, NUMBER OF REFLECTIONS TO BE EXCLUDED FROM LATTICE

REFINEMENT; IF -VE, -NUMBER OF REFLECTIONS WHOSE RASTERS

ARE TO BE DISPLAYED

13.  NELH(I),NELK(I),I=1,8

UP TO 8 INDICES OF REFLECTIONS TO BE EXCLUDED FROM REFINE; OR

UP TO 8 INDICES OF REFLECTIONS TO BE DISPLAYED;

OMIT THIS CARD IF NELIM=0

INPUT AND OUTPUT*************************

UNIT 1 : INPUT DENSITOMETERED FILM ARRAY, any size

UNIT 2 : OUTPUT OF INTEGRATED BACKGROUND SUBTRACTED INTENSITIES.

UNIT 3 : INPUT RADIAL DENSITY CURVE FOR BACKGROUND CORRECTION.

UNIT 4 : INPUT Y-AXIS DENSITOMETER DRIFT CURVE FOR BACKGROUND CORRECTION.

UNIT 5 : INPUT DATASTREAM.

UNIT 6 : OUTPUT DATASTREAM.

UNIT 8 : PLOTTER OUTPUT OF SPOT POSNS. AND C.OF G. DEVIATIONS.

UNIT 9 : INPUT header of image file with autoindexed lattice parameters

4.  MERGEDIFF:              last update 15-Apr-1992

-------------

FOR MERGING HIGH RESOLUTION ELECTRON DIFFRACTION

DATA IN THREE DIMENSIONS.

VX 1.2  15.4.92 RH      HKREV bug removed in PCF output.

THIS VERSION IS SUPPOSED TO CARRY OUT ALL ACTIVITIES RELATED TO HANDLING

ELECTRON DIFFRACTION INTENSITY DATA FROM 3D (TILTED) E.D. PATTERNS IN

ANY TWO-DIMENSIONAL SPACE GROUP.  IT CAN :-

1.  READ IN AND PRODUCE A SORTED, MERGED LIST OF ROUGHLY SCALED DATA

FROM ANY NUMBER OF FILMS, EITHER INDIVIDUALLY AND/OR FROM A

PREVIOUSLY MERGED LIST.

2.  FIT CURVES TO THE MERGED DATA AND PLOT THE WHOLE THING LATTICE

LINE BY LINE, WITH THE POINTS,THE FITTED CURVE AND ITS S.D.

THIS OPTION IS CURRENTLY DONE BY SEPARATE PROGRAM 'SYNCFIT'

TAKING AS INPUT THE DATASET PRODUCED BY MERGHIGH ON UNIT 3

3.  PERFORM PROPER SCALING (S.F. AND TEMPF) AGAINST A PREVIOUSLY

CALCULATED SET OF CURVES.

4.  REFINES TILTAXIS AND TILTANGLE AGAINST PREVIOUS CURVES

5.  DETERMINES AND CORRECTS TWIN PROPORTIONS IN P3 CRYSTALS

4.  THE INPUT IS TYPICALLY EXPECTED TO COME FROM PICKAUTO (E.D. PATT.

PROCESSING PROGRAM) AND OUTPUT TO GO STRAIGHT INTO THE NORMAL

CRYSTALLOGRAPHIC SYSTEM.

############################

CARD INPUT

0  NSER,TITLER (SER NO. AND TITLE FOR THIS JOB)          (I10,10A4)

1  LIST,SORT,NFILE3,NFILE4,IGUIDE,LSUMMARY (IO CONTROL)      (*)

2  LREF,LCURVES,BCURVES (REFERENCE DATA SPECS)	         (*)

3  SCFDET,TA_REFMT,DETWIN,MASK,LMODMSK (OPERATIONAL OPTIONS) (*)

4  ISPGRP,ALNG,BLNG,CLNG,WIDTH,ANG,RRESMAX (STR PARMS)       (*)

5  LCF,LMODSIG,ORESMIN,ORESMAX,

AIMIN,AIMAX,DCUTOFF,LPRINT,CCRMAX           (*)

5a  TITLE IF LCF IS TRUE

FOR EACH FILM TO BE PROCESSED...

(i)    IFILM                                                 (*)

(ii)   NIN,TAXA,TANGL				         (*)

(iii)  SCALE,LBISO,TFPAR,TFPERP,

SGNXCH,ROT180,HKREV,WIN,RESIN		         (*)

(iv)   MASK(1),MASK(2),MASK(3),MASK(4),TP1,TP2,TP3,TP4       (*)

(     LFILM,TITLET                                   (I5,18A4)

(

ON NIN (     IH,IK,I,DELI                                   (2I5,2I6)

(

(     IH>=900  ---> ENDS THE SET OF REFLECTIONS.

IFILM<0  ---> ENDS DATA INPUT

##############################

NSER   - SERIAL NUMBER AND TITLE OF THIS RUN, APPLIED TO

TITLER - OUTPUT ON UNITS 3,4,6, AND 8,10(WHEN WRITTEN).

FLAG MATRIX:

FLAG/ SCFDET  DETWIN TA_REFMT LCURVES  LREF

0		F	F	F	F	F

1		F	F	F	F	T

2		T	F	F	T	T

3		T	F	T	T	T

4		T	T	F	T	T

5		T	T	T	T	T

FLAGS: (T/F)

LIST    : Program lists each reflection on input

otherwise only merged output on unit 3,4 is generated.

SORT    : If F, no sorting and suppression of all outputs.

Note that this flag supercedes all others.

NFILE3  : Sorted reflections output to FOR003.

NFILE4	: Final list of combined reflections output to FOR004.

IGUIDE  : Add guide points to output where curves extend to

higher resolution than data to be merged.

LSUMMARY: Produce a summary file of merged film data.

SCFDET	: Scale and temperature factor determination

TA_REFMT: Tilt axis/angle refinement

DETWIN	: Detwinning treatment (P3)

MASK	: Global mask for determining twin proportions

LMODMSK	: Alter mask if one twin type < 0.0

LREF	: Use reference data for comparisons

LCURVES	: Data read in from curve list vs merged list

BCURVES : Use binary curves as opposed to ASCII encoded ones

ISPGRP - NUMBER OF SPACE GROUP AS BELOW

ALNG   -  A AXIS IN ANGSTROMS

BLNG   -  B AXIS IN ANGSTROMS

CLNG   -  C THICKNESS IN ANGSTROMS

WIDTH  -  WIDTH OF UNIT CELL (ZSTAR COMPARISONS FOR SCALING)

ANG    -  ANGLE BETWEEN A AND B - ONLY FOR P1

RRESMAX-  MAXIMUM RESOLUTION CUTOFF FOR REFLECTION

CONSIDERATION (Angstroms)

LCF    - Produce pre-LCF file for difference Fourier on

channel 8 (T/F)

LMODSIG- Modify SIG values for output (increases lower limit)

ORESMIN- Associated minumum resolution cutoff

ORESMAX- Associated maximum resolution cutoff

AIMIN  - Associated minimum intensity

AIMAX  - Associated maximum intensity

DCUTOFF- Difference amplitude cutoff

LPRINT - Print Delta F's for low angle correl calc

CCRMAX - Correlation calc max resolution limit

Then for each input film:

IFILM  - INTEGER FILM IDENTIFIER

TITLE  - DESCRIPTION OF FILM

NIN    - UNIT NUMBER

TAXA   - ANGLE BETWEEN TILT AXIS AND A AXIS

MEASURED IN DIRECTION OF A TO B BEING POSITIVE

I.E. IF TAXA>0 THEN THE TILT AXIS IS CLOCKWISE

FROM A.

TANGL  - TILT ANGLE IN DEGREES

SCALE  - MULTIPLIED BY AMPLITUDES BEFORE COMBINATION

IF EQUAL TO 0 SCALING IS AUTOMATIC.

LBISO  - (T/F) flag for iostropic temperature factors.

TFPAR  - Temperature factor to be applied to input data.

TFPERP - Temperature factor to be applied to input data.

PAR is parallel to tilt axis, PERP is

perpendicular.  If isotropic temp factor

desired, only TFPAR used.

SGNXCH - IF T, FLIP AROUND A AXIS(T/F). USEFUL IN P121.

ROT180 - IF T, ROTATE 180 DEGREES ABOUT Z-AXIS(T/F).

USEFUL IN P1,P3 -- ONLY APPROPRIATE FOR IMAGES.

HKREV  - IF T, EXCHANGE H AND K AXES(T/F). THIS IS A COSMETIC

FEATURE TO FACILITATE INDEXING OF DIFFICULT TILTED

FILMS. NOTE THAT TLTAXA AND TLTANG MUST BE CORRECT

W.R.T. THE ORIGINAL DEFINITIONS OF THE

DIRECTIONS OF H AND K ON THE FILM.

WIN    - ZSTAR RANGE WITHIN WHICH SPOTS ARE COMPARED

FOR SCALING

MASK   - Four values for detwinning mask.

BBIN   - Four values to specify twin proportions.

Then in each reflection file:

LFILM  - FILM NUMBER

TITLET - TITLE ON UNIT NIN (DATA). IF LFILM IS NOT EQUAL TO

IFILM, THE SEARCH CONTINUES THROUGHOUT UNIT NIN.

IH     - H INDEX OF REFLECTION

IK     - K INDEX OF REFLECTION

I      - INTENSITY OF REFLECTION

DELI   - DELTA(S.D.) OF REFLECTION

######################################

INPUT IS MADE VIA

UNIT  1 - IF REQUESTED, PREVIOUSLY MERGED DATA LIST

UNIT NIN - NEW FILMS TO BE MERGED, NORMALLY UNIT 2

UNIT  5 - CONTROL DATA

UNIT  7 - PREVIOUSLY FITTED CURVES, STORED IN ASCII, IF REQUESTED

UNIT  9 - (logical unit BCURVES) PREVIOUSLY FITTED CURVES IN BINARY FORM

OUTPUT IS MADE VIA

UNIT  3 - MERGED DATA POINTS IF REQUESTED (H,K,Z,INTENS,DELTA,FILMN)

UNIT  4 - FINAL LIST OF COMBINED REFLECTIONS (FOR EXAMINATION)

(SIMILAR TO UNIT 3, BUT CLEARER FORMAT)

UNIT  6 - LINEPRINTER

UNIT  8 - PRE-LCF FILE IF DESIRED

UNIT 10 - DELTA F VS S FORMATTED DATA, PRODUCED IF LCF=T

UNIT 11 - Summary of film data, produced if LCF=T

UNIT 12 - Summary of merged film data, produced if LSUMMARY=T

5.  AVRGDELTAF:

--------------

PROGRAM TO OBTAIN AVERAGE deltaf's starting from DELTAF outputs from MERGEDIFF

VX 1.3      11.4.92       RH.   More films.

input	CARD 1.    NFNAT, NFDER     -  Number of films for native and derivative

to be averaged and subtracted.

NFNAT can be zero.

CARD 2.    ZMIN, ZMAX       -  minimum and maximum zstar for average.

CARD 3.    N1....NN         -  Fortran unit numbers for native data

CARD 3A.   WN1...WNN        -  Weights for native data (can be all 1.0)

CARD 4.    D1....DN         -  Fortran unit numbers for derivative data

CARD 4A.   WD1...WDN        -  Weights for derivative data.

output -  will be to UNIT 3, same format as input with 5 dummy numbers at end

to maintain compatibility with Tom Ceska's EDLCFKPOS program.

6.  SYNCFIT:

-----------

VAX VERSION  SEPT 82

FIT SYNC FUNCTIONS TO LATTICE LINES PRODUCED BY MERGHIGH

WITH OR WITHOUT PREPARATION OF INPUT DATA SET FOR PLOTTING

VERSION INCLUDES ESTIMATES OF ERRORS IN SYNC FUNCTION COEFFS

AND ERRORS IN FITTED CURVE.  ALSO CALCULATES MERGING R-FACTORS

UNIT 2 USED FOR FITTED CURVES

THIS VERSION DOES NOT ELIMINATE NEGATIVE SYNC FUNCTION COEFFS

MISSING REGIONS OF THE LATTICE LINE ARE FILLED IN AS GUIDE POINTS

DATA SETS USED ARE:-

(1)  LATTICE LINE DATA POINTS FOR CURVE FITTING

FORMAT OF DATASET:- TITLE RECORD,I10,10A4;

H,K,ZSTAR,AMP,DIFF,IFILM (2I3,F7.4,2F8.1,I6)

TERMINATED 100,100 ETC

(2)  DATASET FOR NEW FITTED CURVES

FORMAT (2I3,F7.4,F8.1); TERMINATED 100,100

(3)  OUTPUT FORMATTED LIST OF SYNC FUNCTION COEFFICIENTS

(10) PLOT DATA SET READY FOR PROGRAM PLOTCRVS

BINARY DATA SET- SEE SUBROUTINE POINTS FOR CONTENT OF THIS

DATA CARDS ARE

(1) IPLOT     (*)

IF IPLOT=0, FITS SYNC FUNCTIONS AND COMPUTES R-FACTORS ONLY;

IF IPLOT=1, FITS SYNC FUNCTIONS, COMPUTES R-FACTORS, AND

PLOTS DATA POINTS AND FITTED CURVES

(2) ACELL,BCELL,GAMMA       (*)

2D UNIT CELL DIMENSIONS

(3) RESMAX,BSINC,SIGMIN,SIGMAX,SIGUID     (*)

RESMAX; DATA DISCARDED IF (1/D**2) > RESMAX (IN ANGS**-2)

BSINC;  APPLIES TEMP FACTOR EQUIVALENT TO 2*BSINC TO

SINCS IN CURVE FITTING

SIGMIN; MINIMUM SIGMA FOR CURVE FITTING

SIGMAX; MAXIMUM SIGMA FOR CURVE FITTING

SIGUID; SIGMA FOR GUIDE POINTS SET TO SIGUID*(AVERAGE

SIGMA OF NEAREST DATA POINTS). SIGMA IS DOUBLED

AS GUIDE POINTS GET FURTHER FROM DATA POINTS.

ALL THESE HAVE DEFAULT VALUES IF SET AS 0.

7a.  MKLCF:      Standard CCP4 program to create LCF files from formatted data.

---------

7b.  F2MTZ:      Standard CCP4 program to create MTZ files from formatted data.

---------

8.  UPDTXTRA:

------------

PROGRAM TO UPDATE EXTRA INFORMATION IN HEADER TO INCLUDE LATTICE VECTORS FOR

ELECTRON DIFFRACTION PATTERNS

15-OCT-1984   T. CESKA	Original program.

INPUT FILES:

LAT		TEXT FILE CONTAINING LATTICE VECTORS

USUALLY FROM AEDDSP PROGRAM

IN		DIFFRACTION PATTERN IMAGE FILE

Output is four numbers put into the extra header records, locations 4-7.