3-DIMENSIONAL HELICAL PROGRAMS:

-------------------------------

1. HLXSEARCH Determine tilt and origin.                        (RAC)

2. HLXDUMP   Dumps layer line data from transform              (RAC)

3. HLXFIT    Orientations & origins of different particles     (RAC)

4. HLXAVG    Averages data from different particles            (RAC)

5. HLXFOUR   Fourier program                                   (RAC)

6. HLXPROJ   2D filtered image from layer line data            (RAC)

7. EM2FOLD   Imposes twofold normal to axis                    (RAC)

8. HLXSEPDAT Feeder for HLXSEPR                                (RAC)

9. HLXSEPR   Separates overlapping Bessel functions            (RAC)

10.HLXLLOUT  Graphs standard layer line data                   (RAC)

11.HLXSIMUL  Simulates image of helix                          (RAC)

12.HLXFITT   Orientations & origins of different particles     (UNWIN)

1. HLXSEARCH:

-------------

The purpose of this program is to determine the tilt angle of the

helix axis out of the plane normal to the direction of view and at the

same time to position the phase origin on the particle axis.

The user feeds the program with values of amplitude and phase for

pairs of peaks in the computed transform.  It then computes the sum of

F*(phi1-phi2) over these pairs of related reflections.  Values of

(phi1-phi2) are corrected to allow for odd Bessel orders.  They are also

corrected for specified values of phase shift and tilt angle.  The sum

for each combination of phase shift and tilt angles is normalised by

dividing by 360n and then scaled from 0 to 90. The normalised sums are

output as an array.

Different indexing schemes may be tested in a single run, as

described below. A separate residual array is output for each scheme.

Data Cards:

TITLE                  (20A4)

OMEGAM, DOMEGA, NOMEGA, EXM, DEX, NEX, NNLAY, NOP, IDIM1, IDIM2   (*)

Y, D, F, DPHI     (*)     One card for each pair of reflections

NLAY(I), I=1,NNLAY    (8I5)   One card for each pair of reflections

Pairs of cards 3 & 4 are repeated for each pair of reflections

OMEGAM = Beginning value of tilt angle in degrees

DOMEGA = Tilt angle increment in degrees

NOMEGA = Number of points in tilt search (l.e. 25)

EXM = Beginning value of x-shift

DEX = x-shifts increment

NEX = Number of points in x-shift search (le.25)

NNLAY = Number of different selection rules to be tried (le.12)

NOP = Number of pairs of reflections (le.50)

IDIM1,IDIM2 X,Y transform dimensions

Y = Layer line spacing in transform Y grid steps

D = Spacing between reflection pair in transform X steps

F = Average value of amplitude (arbitrary scale) for the pair of

reflections

DPHI = (phase of left)-(phase of right) side reflection in degrees

NLAY = NNLAY Bessel orders for the layer line in question.

2. HLXDUMP:

-----------

Extracts lines of F and phi from transforms of helical particles

produced by FFTRANS. The user specifies the location of the layer line

in reciprocal space, its length and sampling frequency.

In general, the user will wish to have data at points in reciprocal

space which do not correspond to the grid points calculated by FFTRANS,

HLXDUMP assigns values to these points by bilinear interpolation.  The

transform is phase-shifted to bring its origin to the geometric centre

of the box transformed (this position is stored on the disc area by

BOXIM) before any interpolation is carried out. The results of the

interpolation are of course only valid if the original sampling in

reciprocal space is fine enough for the phases of the centred array to

change slowly from point to point.

The program also corrects for tilts of the helix axis out of the

plane normal to the direction of view.  The effect of such a tilt is to

intersect the cylindrical rings of Gn,l(R) on a chord not passing through

the origin. As a result, Gn,l(R) are missed for very small values of R

(this is called the eclipsed region). Gn,l(R) for larger values have the

correct amplitude but the wrong phase; in addition the radii have been

wrongly assigned. The program corrects the phase and reinterpolates along

the line at evenly spaced intervals (selected by the user).

It is often found that the layer lines do not have their symmetry

axis on the line through the origin which is normal to the layer lines.

This problem seems to arise from a shear deformation of the object which

manifests itself as a skewness of the transform. The user determines the

angle of skewness of the pattern and sets SHEAR. The program centres the

lines accordingly.

In addition, ORIGX, ORIGY often does not correspond to the projected

axis of the particle. The user can bring the phase origin to the axis by

adjusting XSHFT &/or YSHFT, which cause translations of the phase origin

parallel to &/or normal to the layer lines respectively.

Note:

Near side means side nearest to the observer of the densitometered

array. For this side, reflections from right-handed helices (positive n)

are in the top left quarter of the transform, reflections from left-handed

helices (negative n) in the top right quarter. Which side it corresponds

to physically depends on the orientation of the film during densitometry

By our conventions, the side of the particle next to the carbon support

film is nearest to the electron beam, and the e.m. film is printed and

densitometered emulsion up (same orientation as in the microscope during

exposure), so the observer is put in the position of the electron beam,

looking at the particle through the carbon.

Data Cards:

TITLE (10A4)

LLTOT,SAMPL,XSHFT,YSHFT,DELBR,ROT,SHEAR,TILT  (*)

RMIN,RMAX,IDUMP,MERID,IZERO  (*)

LLNO,NORD,YSP  (*)

NL1,NL2,NR1,NR2  (*) This record only read in if IDUMP.GE.1

Items 4 (and 5 for IDUMP.NE.0) repeated for each layer line

TITLE identifies the particular choice of data.

LLTOT is the number of separate layer lines to be output.

SAMPL  real space sample size in angstroms

XSHFT, YSHFT  X & Y phase origin shift in real space steps.

DELBR - the required spacing between points at which the layer line is

sampled, in reciprocal Angstroms.   DELBR=0.0004 gives one sample/mm.

on graphical output on printer.

ROT is the angle of the layer lines relative to the horizontal axes

(in degrees).  The angle is positive if the layer lines slope up to the

right.

TILT  tilt of the helix axis (in degrees), calculated by HLXSEARCH

SHEAR is the angle (in degrees) which the transform meridian makes

with vertical axes. (Positive if meridian to left of vertical).

N.B.  Set equal to ROT if transform not sheared.

RMIN, RMAX  limits for layer line plotting in reciprocal angstroms.

Maximum range -0.2 to 0.2 rec Ang

IDUMP  = 0  output graphed only

= 1  output graphed & dumped to disc.

= 2  output dumped to disc only

MERID  = 0  both halves of meridional layer lines to both sides files

= 1  only one half of meridionals to each side

N.B.  Set MERID=0 if you are going to use HLXSEPR to separate

overlapping Bessels.

IZERO  = 0  dumps all layer line data

= 1  dumps only layer line data where NL1, NL2 or NR1,NR2

non zero

LLNO  layer line number

NORD  bessel order for the layer line.

YSP  Y intercept (in Y transform steps), where layer line crosses

vertical axis.

NL1, NL2, NR1, NR2 specify the region of the line to be output. They

are measured in units of DELBR from the layer line origin. NL1, NL2

are the nearest and furthest points from the origin, of the region

of layer line to be output from the left-hand side. NR1, NR2 are the

nearest and furthest points from the origin, of the region to be

selected from the right-hand half of the layer line.

Note that the meridian corresponds to NL1=NR1=0.

FORMAT OF HLXDUMP OUTPUT:

HLXDUMP automatically separates the layer lines into halves and sorts

the halves out into near and far side data, according to the signs of

the NORD values.  The output, which is read by HLXFOUR etc, has the

following format:

TITLE, LLINFO, WTFAC, NORD, LLNO       (10A4,10A1,F10.3,2I5)

R, F, PHI                    (3E10.3)

The last record is repeated for each point along the layer line.

The list of points is terminated by a blank record (i.e. R=0.0)

However, R=0.0 is not a terminator if it is on the first record after

the title record.

Thus, if there are no points for a particular half layer line,the output

at this position will consist of three records; the title record,

followed by two blank records.

3. HLXFIT:

----------

Program to determine the relative orientations of two different helical

particles, by comparing the phases of corresponding points along the

layer lines.  The best relative radial scaling of the two images can also

be determined.

The 'residual' calculated is:

sqrt( sum(F*(phi1-phi2')2)/sum(F) )

where phi2' = phi2 - n dphi + 2pi Z dz

which is tabulated for each angle of rotation (dphi) about the z-axis,

each axial shift (dz) and each radial scaling factor applied to the data

for particle 2.  Any data absent from the data set for particle 2, but

present in the reference data set (particle 1) adds a penalty of F1*(90)2

to the residual sum, if IMISS is set to 1 (otherwise, missing data is

simply ignored).

If particle 2 is thought to be upside-down relative to particle 1,

phi2 is negated.

Data Cards:

FILREF  (A)

LLMAX, CANG, DELBR,  IPRINT, IMISS  (*)

WT(L), L=1,LLMAX (*)

FILIN  (A)

TITLE                                  (20A4)

ISIDE, IPOLE (*)

PHIMIN, PHIMAX, DPHI (*)

ZMIN, ZMAX, DZED (*)

RSCMIN, RSCMAX, DRSC (*)

Items 4 to 9 may be repeated as many times as required, so that several

data sets may be compared with the same reference (particle 1).

Note that each data set should have the same number of layer lines, in

the same order.  Dummy data should be inserted for missing layer lines

(Title record + 2 blank records - see specification for HLXDUMP output).

FILREF file name for the reference particle.

LLMAX is the number of layer lines to be read in (le. 30)

CANG  (in Angstroms),  is the axial repeat

DELBR is the spacing of sample points along the layer lines (reciprocal

angstroms).

IPRINT if = 1 layerline data printed ,if = 0 data not printed

IMISS  if = 1 missing data penalty imposed, if = 0 missing data ignored

WT(I) weighting factors applied to the layer lines..LLMAX of them

FILIN  file name for dataset 2, 3 etc.

TITLE for output to line printer

ISIDE = 0 if data 1 & data 2 both correspond to the same side (near or

far) of the helix.

= 1 if they differ.

IPOLE is set to 0 if particles are to be compared in their initial polar

orientations.  If particle 2 is to be turned upside-down, set IPOLE = 1.

Note that this inversion is carried out first, before any rotation about

the helical axis or translation along the axis.

PHIMIN, PHIMAX specify the range of angles through which particle 2 is

to be rotated about Z (anticlockwise rotation, when viewed down the axis

corresponds to positive rotation angle).

DPHI the size of the steps between successive rotational orientations

in which particle 2 is to be compared with the reference.Up to 12 steps

ZMIN, ZMAX, DZED  specify (in Angstroms) the range and step size for the

translations parallel to z which are to be tried. Up to 12 steps allowed

RSCMIN, RSCMAX define the range of scaling factors to be applied to

distances along the layer lines of particle 2 data:

R-distance of peak in transform 2

R-distance of same peak in transform 1

DRSC is the step size in the radial scaling search - up to 10 steps.

4. HLXAVG :

-----------

Program for averaging different sets of layer line data. (Note :

HLXFOUR is able to average the two sides of a single particle).  The

relative orientations of different particles are first determined using

HLXFIT, which also supplies amplitude scaling factors if required.

Data for a difference Fourier can be produced, using scaling

factors of appropriate signs.

Data sets can be modified for various purposes, e.g. changing the

sampling distance (DELBR) of the layer lines, or changing the radial

scale.

Input layer lines on FILIN specified below

Ouput averaged layer line data on logical LLOUT specifed in setenv

Data Cards:

TITLE  (10A5)

LLMAX, NSIDE, CANG, DELBR, IPRINT  (*)

FILIN (A)

ISIDE, IPOLE, PHISHF, ZSHIFT, RSCALE  (*)

FSCAL(L), L=1,LLMAX                  (*)

items 3 to 5 are repeated for each data set (half transforms ?) to be

included in the average.

Note that each data set should have layer lines in the same order, with

no gaps. Dummy data should be inserted for missing layer lines (Title

record, followed by 2 blank records - see specification for HLXDUMP

output).

LLMAX is the total number of layer lines to be averaged (le.40).

NSIDE is the total number of data sets to be averaged.

CANG   (in angstroms) is axial repeat distance

DELBR  spacing of points sampled along the averaged layer lines -

usually the same as the spacing in the individual data sets.

IPRINT if = 1 prints layer line data,otherwise does not

FILIN  file name for input data set

ISIDE is set to 0 for 'near-side' data, to 1 for 'far-side' data.

IPOLE = 0 if data set is to be included in its initial orientation, = 1

it is to be turned upside-down before addition.

ZSHIFT and PHISHF define the shift (in A) in the positive z-direction &

the rotation angle (in degrees) about the z-axis to be applied to the

data set to bring the particle into the required common orientation.

RSCALE is a radial scaling factor to be applied to all radial distances

along the layer lines, to bring all the data sets to the same size scale

FSCAL are relative scaling factors for the amplitudes on each layer line

in case different data sets are on different scales etc.

5. HLXFOUR:

-----------

Program to carry out a Fourier-Bessel transformation of layer line

data to produce a three-dimensional density map, which may be output as

sectiions of various types.

The transformation is described by Klug, Crick & Wyckoff (Acta

Cryst. 1968, 11, 199) and their conventions are adhered to - e.g. a

positive value of n corresponds to a right-handed helix.

rho(r,phi,z) = 1/c * Sum over l & n (gnl(r)*exp(i(n*phi-2pi*l*z/c))

where

gnl(r) = exp(-i*n(PHI+pi/2)) * integral (Fl(R) * Jn(2pi*R*r) 2pi*R*dR

The input values of the transform (HLXDUMP output) are implicitly

assigned to the plane PHI = +- pi/2 , where PHI is measured from the

X-axis.

Input-output streams:

Input stream  1      layer line data           LLIN

Output stream 2      3-D map                   OUT1

Output stream 3      Projection                OUT2

Output stream 4      File of g's for plotting  GOUT

Data Cards:

TITLE (10A4)

C, DELBR, RMAX, DELSR  (*)

LLMAX, ISIDE, KOUT  (*)

WT(L), L=1,LLMAX (*)

IHFH, IHFC, IHFV, IHFP, IHFZP, IQATOR, MAP, ITRFN   (*)

Data for an artificial mean radial density distribution (g00(r)) is

inserted here, if IQATOR = 1.

Followed by one or more sets of output parameters, in the following

order :

PHIMIN, PHIMAX       (*)        if IHFH = 1

ZMIN, ZMAX, DELZED   (*)        horizontal sections, perpendicular to z-

----------------------------------

RMIN, RMAX           (*)        if IHFC = 1

PHIMIN, PHIMAX, DELPHI (*)  cylindrical sections, parallel to z-axis.

ZMIN, ZMAX, DELZED   (*)

----------------------------------

RMIN, RMAX           (*)        if IHFV = 1

PHIMIN, PHIMAX, DELPHI (*)  vertical central sections at successive

angles

ZMIN, ZMAX, DELZED   (*)

----------------------------------

XMIN, XMAX, DELX     (*)        if IHFP = 1

PHIMIN,PHIMAX        (*)    vertical parallel sections of constant x.

ZMIN, ZMAX, DELZED   (*)

----------------------------------

PHIMIN, MIND, DMIN  (*)      if IHFZP = 1

ZMIN, ZMAX, DELZED   (*)   single projection down z-axis.

C is the axial repeat distance of the helix in Angstroms.

DELBR is the spacing between samples along the layer lines, in

reciprocal angstroms.

RMAX is the maximum radius for which little g's are calculated

DELSR is the spacing of points in the map,in the y direction. Points in

the x direction are calculated at intervals of 2/3 DELSR, if IPLOT is

set to 2, to be compatible with PLOTFOUR. If IPLOT = 0, samples in the

x direction are at intervals of 5/6 * DELSR to give isometric output on

the line printer (8 lines per inch).

LLMAX total number of layer lines read in

WT(L) weighting applied to each layer line (LLMAX of them)

NSP input unit number (layer line data)

ISIDE = 0 for nearside data, = 1 for farside data

IABS(KOUT)  = 1 gives isometric line printer output only,

= 2 gives output map in standard format for TONE etc

= 3 gives output in standard format and line printer

If KOUT negative get listing of big G's

MAP   = 1 outputs file of little g's for plotting with HLXGOUT

ITRFN .ne. 0  is supposed to apply a transfer function correction. NOT CHECKED!

RMIN,RMAX limits of radius in angstroms,RMAX must not exceed RMAX on

item 2.

PHIMIN,PHIMAX limits in degrees of sector of map to be output

DELPHI spacing in degrees over which PHI is stepped

ZMIN, ZMAX are the axial limits for the map to be output

DELZED is the spacing in the axial direction of points in the map.

if IHFC=1, RMIN & RMAX are the inner and outer radial limits of the map.

RMAX in this case should not be more than on card 2

6. HLXPROJ:

-----------

Reverse transforms selected layer line data, output by HLXDUMP,

to synthesize a filtered projected image. The particle may be rotated

about the helix axis to obtain views different from the original. The

filtered images are output as numerical arrays on disc for plotting

Data Cards:

TITLE           (20A4)

LLTOT, C, DELBR  (*)

RMAX, DELSR, IRSTEP, IJUMP, NV  (*)

WT(L), L=1,LLTOT (*)

PHI(IV), IV=1,NV (*)

ZMIN, ZMAX, DELZED, DMIN  (*)

FILIN (A)

LLTOT = number of separate layer lines in a single data set.

C = axial repeat of helical particle

DELBR = spacing (dR) of data points along layer lines.

RMAX = maximum radius of particle

DELSR = radial separation between calculated density points in real

space.

IRSTEP = intervals (in steps of DELBR) at which data are to be included

the calculation.

If = 1, every point along the layer line is used,

if = 2, alternate points are included, etc.

The calculation may be very time-consuming if all the layer

line points are used to calculate points in a large filtered

image array, and layer lines are often sampled much more

finely than necessary, especially for this purpose.

IJUMP = option control:  outputs one-sided filtered images for negative

values,double-sided images for positive values.

+-1, prog thinks it is receiving 'near-side' data,

+-2, prog thinks data is 'far-side',

+-3, prog expects 'near-side' followed by far-side, on 2

separate files.

NV = number of different filtered images to be calculated from the data

set showing views from different angles around the helix axis.

WT(L) = weighting factors for each layer line.

PHI(IV)'s are the required view angles in degrees.

ZMIN,ZMAX,DELZED = lower and upper values of axial distance z required

for the filtered image, and the interval between sample

points.

FILIN file name of layer line data set

Note : RMAX/DELSR * (ZMAX - ZMIN)/DELZED must not excced 50000

up to 40 layer lines

up to 200 steps / layer line

up to 20 views

7. EM2FOLD:

------------

This program is no longer available. If you need a program to search

a 2-fold, please contact jms@mrc-lmb.cam.ac.uk as the program hlx2fd

may be sent to you on request.

8.HLXSEPDAT

-----------

This is a feeder program for HLXSEPR which copies layer line data

from different views in known orientations(found in HLXFIT) to a direct

access area. It assumes that the layer line data are sampled at the

same spacing in each view.

Inputs on stream 2 with filenames specified in data

Output to direct access area on stream 3 specified in data

Data cards :

FILOUT (A)

NVIEW,LLMAX,DELBR,C,IREST   (*)

FILOUT Filename of output direct access data set

NVIEW  No. of "sides" to be input for this run

LLMAX  No. of layer lines for each side

DELBR  Layer line sampling in rec. Ang.

C      Helical repeat in Angstroms

IREST  0  Not restart ie Starting new direct access area

1  Restart     ie Adding data to existing area

Then for each side :

FILIN  (A)

PHI,XSHIFT,YSHIFT,IPOLE  (*)

FILIN  Filename of data set containing layer line data for this

side.

PHI    Angle of view in degrees  For reference particle near

side has PHI=270, far side has PHI=90. Relative values

of PHI found in HLXFIT then add to these. e.g. If

HLXFIT gives a value -10 degrees for a second particle,

corresponding PHI values for near and far sides will

be 260 and 80 respectively.

XSHIFT Extra x-shift of origin in angstroms. Generally zero.

YSHIFT y-shift in angstroms. Used to refer all particles to

common origin.  Use value given by HLXFIT

IPOLE  Used to invert particle in z-direction, if HLXFIT

indicates this is necessary.

9.HLXSEPR:

---------

Program to separate overlapping Bessels , using views of helix

in different orientations. Uses LINFIL least squares package. The

separated Bessel functions produced correspond to a "NEAR SIDE"

set of transform values, ready to go into HLXFOUR.

Input layer line data on direct access file 2 , created by

HLXSEPDAT program.

Output separated Bessel functions on stream 3 , in HLXFOUR

format. Plot graphs of separated Bessels on printer.

Data cards:

FILIN (A)

FILOUT (A)

TITLE (10A4)

NVIEW,LLMAX,RADIUS,IGRAPH (*)

(RSCAL(NV),NV=1,NVIEW)  (*)

Then for each layer line:

NL,NORD,(IORD(NO),NO=1,NORD)  (*)

(FITWT(NV),NV=1,NVIEW)        (*)

FILIN  Input direct access data file from hlxsepdat

FILOUT Output separated layer line data for hlxfour

TITLE  Title for separated data.

NVIEW  No. of views (ie. sides)

LLMAX  No. of layer lines in each view

RADIUS Radius of particle in angstroms - used for determining

where Bessel contribution starts in transform.

IGRAPH Draw graphs of separated Bessels if non-zero

(RSCAL(NV),NV=1,NVIEW)  Radial scale factors for each view

from HLXFIT.

For each layer line:

NL     Layer line no.

NORD   No. of different Bessel orders on layer line

IORD(NO),NO=1,NORD  Bessel orders on layer line

FITWT(NV),NV=1,NVIEW   Amplitude weight factors for each view

for this layer line. These are used to multiply the

amplitudes on each layer line in each view before the

separation is done.

10.HLXLLOUT:

-----------

Program to select and graph standard format layer line data either

with autoscaling for each layer line or on constant amplitude scale

Layer line input on stream 1 (LLIN)

Data cards:

TITLE (10A4)

LLMAX,FSCA,FMAX,RMIN,RMAX,CANG (*)

IPLOT(I),I=1,LLMAX (*)

TITLE    Title for plots (up to 40 chars)

LLMAX    No. of layer lines to be input

FSCA     Multiplying amplitude scale factor applied when data read

from disc. Allows data set to be put on absolute scale.

FMAX     Maximum amplitude overall on layer lines to be plotted,

after FSCA has been applied.

If set to zero, then autoscaling used on each layerline.

RMIN,RMAX Radial range for plotting box (rec Ang.)

CANG     Helical repeat (Angstroms) to set ZSTAR

IPLOT(I) I=1,LLMAX  Plot particular layer line if 1, not if 0

11.HLXSIMUL:

-----------

Program to simulate projected image of helix for testing helical

reconstruction programs. Subunits built from arbitrary numbers

of spherical blobs of different size, position and density.

Single start helix specified by helical parameters DZ,DPHI.

Multi-start helix produced by putting more blobs into subunit,

related by appropriate rotational symmetry. The generated piece of

helix is centered in the z direction, which is vertical in TONE.

y axis is horizontal in TONE and x axis comes out towards observer.

Back or front of helix can be cut off and a linear ramp weighting

applied in order to simulate one sided or partially one-sided images.

Output standard image on stream 1 (OUT)

Present dimensions of output image are 256 x 512

DATA CARDS

(1) NBLOB,NUNIT,AXIS,DZ,DPHI,PHIROT,TILT,PBACK,PFRNT,IRAMP (*)

(2) RADH,RADS,Z0,PHI0,WT (*)

(2) repeated NBLOB times.

NBLOB  Number of blobs per subunit

NUNIT  Number of subunits to be generated

AXIS   Y position of helix axis   pixels

DZ     Z rise per subunit         pixels

DPHI   Phi twist per subunit      degrees

PHIROT Rotation of whole helix about z axis  degrees

TILT   Tilt of helix axis out of plane   degrees

Tilts about centre of helix about axis parallel to y.

Positive TILT tilts top of particle towards observer.

PBACK,PFRNT Back and front planes for windowing in pixels.

Blobs lying outside these planes are omitted. If both set

to zero there is no windowing.

IRAMP  0 Simple cut off by windowing planes

1 Cut off by window and linear ramp weighting of

densities from 1 at front to 0 at back of window

-1 Ramp weighting from 0 at front to 1 at back

For each blob in the subunit:

RADH   Radius from axis   pixels

RADS   Radius of blob     pixels

Z0     Z position of blob in first subunit  pixels  (generally zero)

PHI0   phi position of blob in first subunit   degrees

(measured from x-axis)

WT     Density weight

12.HLXFITT:

-----------

Program finds the common origin for a "test" and "reference"

helical object, and calculates the relative amplitude and

radial scales.

The common origin is determined by minimizing the phase

residuals between reference datasets, according to:

phiF = sqrt(sum(|Fref|*dphi**2)/sum|Fref|)

Program also gives R-factors:

Rf = sum(||Fref| - |Ftest||)/sum|Fref|

Two alternative output files may be generated:

(a) new test dataset, oriented and radially scaled to fit

the reference set

(b) "phase error" dataset, like (a) except that the phases

output are the difference phases: phi(r) - phi(t)

equatorial data are omitted.

CNTFILE        - control data file

RDLFILE        - output file listing:

Rf x 1000, phimin, zmin, rscalmin, phiF x 1000

NEWFILE        - new test dataset

Data cards in CNTFILE:

HLXTITLE       - image title

REFILE         - reference filename

NLL            - number of repeats (equivalent to llfact)

rep_dis        - repeat distance*NLL (A)

DELR1, DELR2   - layer-line sampling intervals (ref., test)

ACUT           - amplitude cutoff (% of max. amp. in ref.)

LLSPEC(I), WTSPEC(I) 6(I5.F5.1)

- up to 6 special layer-lines and weights

TFILE          - test filename

ISIDE          - 0 for near, 1 for far side

IPOLE          - 1 to rotate test helix by 180 degrees

PHIMID         - estimated phi value to search about (degrees)

DPHI           - search increment in phi

IPL            - no. of steps in phi (max. 8)

ZMID           - estimated z value to search about (A)

DELZ           - search increment in z

IZL            - no. of steps in z

RSCMID         - estimated rscal to search about

DRSC           - search increment in rscal

IRL            - no. of steps in rscal (max. 8)

IOUT           - 1 to write out NEWFILE