Brief description of the icosahedral virus reconstruction

programs on the Alliant as at 10 Feb 93.

List of programs

refine  -  finds orientations and origins by common lines

x37corr -  cross-correlates two particles of known orientation

to find relative hand and scaling

racmat  -  extracts data from transforms needed for solving for big G's

racbg   -  solves for big G's

raclg   -  converts big G's to little g's

racfb   -  does Fourier summation from little g's to make 3-D map

mapconv -  converts 3-D map to MRC format

**********************************************************************

refine

------

This works from an MRC format transform of the image and gives

three options - a whole asymmetric unit search in angle

- an omega search for fixed theta, phi

- an x,y origin search for specified theta,phi,omega

Data

1) MINR, MAXR, NSAMPL, FMIN

MINR, MAXR   set radial range used in transform steps

NSAMPL       number of samples taken in specified radial range

FMIN         fraction of mean amplitude below which points

are ignored.  Typically use FMIN=1. to throw away

data lower than mean.

2) FNAME            file name for output of results

3) ISWITCH          1 to refine origin

2 to search asymmetric unit

3 for omega search

Remaining control data depend on ISWITCH value

ISWITCH=1

4) THETA, PHI, OMEGA   - specifies angle of view in degrees

5) XMIN, DELX, NXSTEP, YMIN, YMAX, NYSTEP

- sets origin search parameters in pixels,

search around current best value of origin

ISWITCH=2

4) IENTER =1           - read current best origin

5) ORIGX, ORIGY        - current best origin in pixels

ISWITCH=3

4) IENTER =1           - read current best origin

5) ORIGX, ORIGY        - current best origin

6) THETA, PHI          - theta, phi in degrees for omega search

Examples of scripts to run different options:  parameters to be

changed from particle to particle are set as parameters and invoked

on command line.

Origin search

#!/bin/csh -f

#

setenv IN v$1.trn1

/ss0/rac1/icos/refine.exe << eot

6,25,20,1.0

v$1.org

1

$2,$3,$4

$5,0.2,11,$6,0.2,11

eot

Asymmetric unit search

#!/bin/csh -f

#

setenv IN v$1.trn1

/ss0/rac1/icos/refine.exe << eot

6,25,20,1.0

v$1.asu

2

1

$2,$3

eot

Omega search

#!/bin/csh -f

#

setenv IN v$1.trn1

/ss0/rac1/icos/refine.exe << eot

6,25,20,1.0

v$1.om

3

1

$2,$3

$4,$5

eot

*******************************************************************

x37corr

-------

This program cross-correlates two particles of known orientation

and origin (found by refine) using 37 pairs of cross-common lines,

using the transforms of the two particles in MRC format.  The program

finds relative handedness, radial scaling (magnification) and

amplitude weighting of two particles.  The two transform files are

specified as IN1 and IN2 with IN1 being the "reference" particle.

Data

1) THETA1, PHI1, OMEGA1   - orientation of reference particle

2) THETA2, PHI2, OMEGA2   - orientation of second particle

3) FNAME                  - file name for output of scaling info

4) RSCMIN, RSCMAX, DELRSC - range of radial scale factors

5) RMIN, RMAX             - range of transform radii to use

6) NBAND, INCR, NSAMPL    - number of bands for amplitude scaling

- width of each band in transform steps

- number of samples to be taken in each band

7) IENTER =1              - read origin for reference particle

8) ORIGX1, ORIGY1         - origin for reference particle in pixels

9) IENTER =1              - read origin for second particle

8) ORIGX2, ORIGY2         - origin for second particle in pixels

Example of script to cross correlate general particle against particle

v2 as reference

#!/bin/csh

#

setenv IN1 v2.trn

setenv IN2 v$1.trn

/ss0/rac1/icos/x37corr.exe << eot

87.,-9.,167.

$2,$3,$4

v2$1.sca

0.95,1.05,0.01

1.,25.

5,5,5

1

66.,63.4

1

$5,$6

eot

#Now print the temporary output file

smallprint x37corr.tmp

****************************************************************

racmat

------

This program extracts data needed to form normal matrices

for solving for big G's from transforms of particles and stores

it in various files, one indexing file which refers to all particles

and one file for each particle which contains the data.

The indexing file is called  emicomat.dat

Data

1) I        -  1  open new emicobg.dat file for first particle

0  use existing emicobg.dat file for subsequent particles

2) FNAME    -  filename for input transform

3) IENTER  =1    to enter origin

4) ORIGX, ORIGY  x,y origin in pixels

5) TITLE         title for this data set

6) FNAME         filename for big G data for this image

7) LMAX, NSAMP, ICHECK  - LMAX  number of radial annuli to be used

NSAMP sets spacing of annuli as 1./(NSAMP*DIAM)

Usually take NSAMP=2

ICHECK print control, usually 0

8) DIAM          diameter of particle in pixels

9) THETA, PHI, OMEGA, NSYM   angles of view.

NSYM=5 for 5-fold view, 0 otherwise

10) RSCAL       radial scale factor from x37corr

11) NBAND, INCR number and size of bands for amplitude scaling from

x37corr.   If 0,0 then no amplitude scaling.

If specified, then amplitude scale factors follow.

12) AMPFAC      NBAND records containing amplitude scale factors

Example of script for processing two particles, opening new index

emicomat.dat file on first

#!/bin/csh -f

#

/ss0/rac1/icos/racmat.exe << 'eot' > racmat.log

1

v2.trn

1

66.,63.4

v2 hbhiv

cmv2.dat

30,2,0

66

87.,-9.,167.,0

1.

0,0

'eot'

/ss0/rac1/icos/racmat.exe << 'eot' >> racmat.log

0

v3.trn

1

64.7,64.8

v3 hbhiv

cmv3.dat

30,2,0

66

89.,3.,207.,0

1.

0,0

'eot'

******************************************************************

racbg

-----

This program takes the normal matrix data files set up

by racmat and solves the linear equations on each annulus at

each Z height for the big G's.  Reads indexing information

from  emicomat.dat  and produces file of big G's in  emicobg.dat.

Data

1) IBMAX, NBAND, ILIST, SPREAD

-  IBMAX number of radial annuli to solve on.  Usually same as

LMAX in racmat

-  NBAND controls whether to read and apply amplitude scaling

factors.  If NBAND=0, no scaling factors, otherwise

NBAND scaling factors for each image (see 4 below).

-  ILIST controls output with 0 least, 1 intermediate, 2 lots

-  SPREAD sets maximum value allowed for mean inverse eigenvalue

when solving normal equations on each annulus.   If

value exceeded, higher order Bessels are dropped until

equations soluble.   If too many annuli affected, then

need more views.

2) NVIEW    -   number of particles to be included

3) NV(I)    -   index numbers of the NVIEW particles to be included

(used to access the  emicomat.dat indexing file)

4) AMPFAC   -   include only if NBAND.ne.0 !!!!!!!!!!!

NVIEW records each containing NBAND scaling factors

5) FNAME    -   NVIEW records of names of files containing normal

matrix data

Example script combining data from 6 particles, which are the first

6 entries in  emicomat.dat  but which come from images 2,3,4,5,6,7.

No amplitude scaling factors.

#!/bin/csh -f

#

/ss0/rac1/icos/racbg.exe << 'eot' > racbg.log

30,0,0,1.

6

1,2,3,4,5,6

cmv2.dat

cmv3.dat

cmv4.dat

cmv5.dat

cmv6.dat

cmv7.dat

'eot'

*******************************************************************

raclg

-----

Program that converts big G's to little g's.    Takes big G

data from  emicobg.dat  and puts little g's  in  emicolg.dat.

Data

1) TITLE    -  title for this run

2) MAXR, ISMAX, DELSR   -  MAXR  transform cut-off radius in annuli

usually IBMAX (as set in racbg) but can be

reduced for lower resolution

-  ISMAX  number of radial steps in real space

-  DELSR  spacing of points in real space

measured in original pixels of densitometered

image

3) IDUMP                -  usually 0, if 1 opens file for debugging

Example script

#!/bin/csh -f

#

/ss0/rac1/icos/raclg.exe << 'eot' > raclg.log

hbhiv v2,v3,v4,v5,v6,v7 11Jan93

30,33,1.

0

'eot'

*************************************************************************

racfb

-----

This program uses the little g's from emicolg.dat in a Fourier

Bessel sum to produce a three dimensional map in the 5-fold setting.

This map is then rotated to produce maps in the 2-fold and 3-fold

settings (ie 2-fold or 3-fold up z)  Output files are   icos2f.map,

icos3f.map, icos5f.map.    AT PRESENT THESE ARE NOT IN MRC FORMAT

(use  mapconv  to convert to MRC format).

Data

1) NHAND      0  handedness unchanged,  1  change handedness

Example script

#!/bin/csh -f

#

/ss0/rac1/icos/racfb.exe << 'eot' > racfb.log

0

'eot'

**********************************************************************

mapconv, mapconv3

-------  --------

Convert 2-fold or 3-fold maps to MRC format.   Expects input

map on icos2f.map (icos3f.map) and puts output map on OUT

#!/bin/csh -f

#

setenv OUT hbhiv_2fold.map

/ss0/rac1/icos/mapconv.exe