ROTATIONAL AVERAGING and FILTERING PROGRAMS

-------------------------------------------

1. RFILTIM:

----------

Progam for filtering images with rotational symmetry. Using the

Fourier transform of the image, the program finds the best position for

the postulated rotational symmetry axis and then decomposes the transform

into cylindrical harmonics about this origin. The program produces a

rotational power spectrum, which is printed and displayed with CURVY.

Those harmonics consistent with the postulated symmetry are then

combined to produce a rotationally filtered image.

(Reference - Crowther and Amos  J. Mol. Biol.(1971) 60,123-130.)

Input   Standard transform on stream 1 (IN)

Output  Filtered image on stream 2     (OUT)

Output  CURVY file on FOR003

CONTROL DATA

(1)  HEADER                                 (10A)

(2)  N,NR,RSTEP,NLR,RSCAL                   (*)

(3)  DXMIN,DXMAX,DELDX,DYMIN,DYMAX,DELDY,NBSTEP,NBMIN  (*)

(4)  ASCAL,BSCAL,PHI0,SCALE,JOPT,LGDUMP,NORM     (*)

N     Rotational symmetry to be tested

NR    No. of annuli of data to be used from the transform

RSTEP Spacing in transform steps of annuli

NLR   No. of radial steps in filtered image

Size of step in filtered image

RSCAL ---------------------------------------------

Size of step in original densitometered image

DXMIN,DXMAX   Range of origin search in x direction measured in

densitometer steps

DELDX         Interval of search in x direction in densitometer steps

DYMIN,DYMAX   Similarly for y direction

DELDY

NBSTEP        No. of transform annuli to be binned into a band when

printing residuals from origin search

NBMIN         No. of bands (of size NBSTEP) to be omitted when

determining best origin.  (First band i.e. low resolution

data, is generally much the strongest and may be biased

by gross stain distribution, in which case it should be

omitted.)

ASCAL,BSCAL   Radial weighting factors in transform which may be used

to increase artificially the strengths of the high

frequencies.  USE WITH CAUTION! Annulus number IR has a

scale factor (ASCAL+IR*BSCAL) applied. For normal use

set ASCAL=1.0, BSCAL=0.

PHI0          The filtered image is rotated PHI0 degrees relative to the

densitometered image. Positive PHI0 rotates the image

clockwise.

SCALE         Densities in the filtered image are multiplied by SCALE.

SCALE=-1. inverts image contrast.

JOPT          0  g0 included in filtered image

1  g0 omitted from filtered image

LGDUMP        0  Do not dump gn's to disc file

1  Dump gn's to disc file (FORTRAN 10)

NORM          0  Do not normalise power spectrum

1  Normalise spectrum with power of N=0 term equal to 1

Current limitations:

NR .LE. 40

NLR .LE. 50

Maximum image size transformed 256x256

Maximum Bessel order 70

Maximum of 15 different Bessel orders included in filtered image

Maximum size of origin search 10x10 positions

2. ROTAV:

--------

Rotational averaging of 2D image or 3D maps about z-axis.

3. IMROTRAN :

-----------

IMROTRAN is a program to rotate/ translate an image file (the relative file)

and calculate a correlation coefficient wrt another image file (the reference

file), then to output another image file of the average the reference file

with the best fit of the relative file.

INPUT DATA:

(40A1) Input image file (reference file does not move)

(40A1) Input image file (relative file is rotated/translated)

(40A1) Output image file (average of above two)

(*)    Theta1, Theta2, Thinc :

starting, finishing angles and increment in

degrees. A +ve angle rotates relative file

clockwise.

(*)    Xtrans1, Xtrans2, Xinc :

starting, finishing translation in X and increment

in pixels. A +ve value translates relative file

to the right.

(*)    Ytrans1, Ytrans2, Yinc :

starting, finishing translation in Y and increment

in pixels. A +ve value translates relative file

upwards.

(*)    Rmax :

maximum radius (in pixels) over which the correlation

coefficient is to be calculated.