GENERAL PROGRAMS FOR PROCESSING MICROGRAPHS

-------------------------------------------

1.  IMCON    P  Get image from CDS dual-port disc, put on VAX disc.

2.  LABEL    P  Image handling, does a few nice things.

3.  FFTRANS  C  Fast Fourier transform.

4.  TRNOUT   C  Output amplitudes & phases to line printer.

5.  TRMASK   C  Masks transform for filtering

6.  INTERPO  P  General 2D re-interpolation program

7.  RFILTIM  C  Rotational filtering

8.  SPLINEFIT P Image straightening using AED

9.  BOXIM    C  Standard boxing program for selection of an area.

10.  BOXIMAGE    Same as BOXIM but does not shift origin.

11.  IMEDIT   P  General editing program for image headers

12.  IMROTRAN    Rotates/translates 2D image.

P = Procedure i.e. Type the name and program will prompt you.

C = Command file which should be edited and entered as batch job.

1. IMCON: Runs online with prompts.

--------

2. LABEL: Runs online with prompts.

---------

3. FFTRANS:

-----------

This program will do 2 dimensional FFTs in either direction.

The realspace origin is at (1,1) and the origin of reciprocal space is at

(1,NY/2+1). The fourier transform of an image NX, NY is NX/2+1,NY complex values

.

Very large images (up to 8192 * 8192) are transformed using the disc based

routine BIGFFT. The switch point is now determined by the available working

set size.Small images are processed in core.

3 dimensional images are treated as image stacks, each section being transformed

independently; only single sections of large images may be used.

No input parameters are required

Input stream IN input image or fourier transform

Output stream OUT output fourier transform or image

Output stream SCRATCH scratch file used only for large images.

4. TRNOUT:

----------

Program to shift quadrant of FFT to produce suitable amplitude & phase

output for the line printer i.e. the transform is continuous across the

meridian. The output is log base 2 (amplitude) on a scale 0 to 9 and

phase is in 10 degree intervals coded A - Z 0 - 9

At present can only put out central 129 columns about F(0,0).

Data input stream 5:

FMAX, IYMAX, XSHFT, YSHFT  (*)

FMAX sets the level above which all amps are set to 9

IYMAX  sets the number of Y lines output.  Maximum 256.

XSHFT, YSHFT X & Y shift for phase origin in grid units

Transform input on stream 1 (IN)

Output amplitudes for TONE on stream 2 (OUT).  NL: if not required.

5. TRMASK:

----------

Program for masking a fourier transform

Data input stream 3:

ISHAPE, IPIXEL  (*)

if ISHAPE = 1 or 2 :

IH(I), IK(I), XC(I), YC(I), RAD(I)  (*)

if ISHAPE = 3:

IH(I), IK(I), XC(I), YC(I), DELX(I), DELY(I)  (*)

ISHAPE  if = 1 hard edge circular holes

if = 2 soft edge circular holes gaussian weighted to EXP(-2) at edge

if = 3 hard edge rectangular holes

IPIXEL pixel size used in TONE display of transform (in 1/100 inch)

If IPIXEL=0, XC etc taken to be transform steps

IH, IK  indices of spot

XC, YC hole centre in mm on TONE output measured from F(0,0) X positive across

page to right, Y positive up page, from pixel centre to pixel centre

RAD  radius for circular holes in mm.

DELX, DELY  half edge lengths for rectangular holes in mm.

Transform input on stream 1 (IN) standard format

Output masked transform on stream 2 (OUT)

6. INTERPO: Runs online with prompts.

----------

7. RFILTIM:

----------

Progam for filtering images with rotational symmetry. Using the

Fourier transform of the image, the program finds the best position for

the postulated rotational symmetry axis and then decomposes the transform

into cylindrical harmonics about this origin. The program produces a

rotational power spectrum, which is printed and displayed with CURVY.

Those harmonics consistent with the postulated symmetry are then

combined to produce a rotationally filtered image.

(Reference - Crowther and Amos  J. Mol. Biol.(1971) 60,123-130.)

Input   Standard transform on stream 1 (IN)

Output  Filtered image on stream 2     (OUT)

Output  CURVY file on FOR003

CONTROL DATA

(1)  HEADER                                 (10A)

(2)  N,NR,RSTEP,NLR,RSCAL                   (*)

(3)  DXMIN,DXMAX,DELDX,DYMIN,DYMAX,DELDY,NBSTEP,NBMIN  (*)

(4)  ASCAL,BSCAL,PHI0,SCALE,JOPT,LGDUMP,NORM     (*)

N     Rotational symmetry to be tested

NR    No. of annuli of data to be used from the transform

RSTEP Spacing in transform steps of annuli

NLR   No. of radial steps in filtered image

Size of step in filtered image

RSCAL ---------------------------------------------

Size of step in original densitometered image

DXMIN,DXMAX   Range of origin search in x direction measured in

densitometer steps

DELDX         Interval of search in x direction in densitometer steps

DYMIN,DYMAX   Similarly for y direction

DELDY

NBSTEP        No. of transform annuli to be binned into a band when

printing residuals from origin search

NBMIN         No. of bands (of size NBSTEP) to be omitted when

determining best origin.  (First band i.e. low resolution

data, is generally much the strongest and may be biased

by gross stain distribution, in which case it should be

omitted.)

ASCAL,BSCAL   Radial weighting factors in transform which may be used

to increase artificially the strengths of the high

frequencies.  USE WITH CAUTION! Annulus number IR has a

scale factor (ASCAL+IR*BSCAL) applied. For normal use

set ASCAL=1.0, BSCAL=0.

PHI0          The filtered image is rotated PHI0 degrees relative to the

densitometered image. Positive PHI0 rotates the image

clockwise.

SCALE         Densities in the filtered image are multiplied by SCALE.

SCALE=-1. inverts image contrast.

JOPT          0  g0 included in filtered image

1  g0 omitted from filtered image

LGDUMP        0  Do not dump gn's to disc file

1  Dump gn's to disc file (FORTRAN 10)

NORM          0  Do not normalise power spectrum

1  Normalise spectrum with power of N=0 term equal to 1

Current limitations:

NR .LE. 40

NLR .LE. 50

Maximum image size transformed 256x256

Maximum Bessel order 70

Maximum of 15 different Bessel orders included in filtered image

Maximum size of origin search 10x10 positions

8. SPLINEFIT: Runs online with prompts

------------

This program displays a filament (from an image file), fits

a cubic spline to it, and then straightens it according to

a choice of two different algorithms.

The first method "shears" the curved filament by generating

new slices with linear interpolation which are shifted with respect

to the original image.

The second method assumes a "normal mode of bending" where

area elements on the inside of a curve are compressed while

those on the outside are expanded.

It calculates the line of density normal to the spline curve

(with bilinear interpolation), tilts them back to horizontal, and

shifts them with respect to the original image to generate a

straight filament.

An output image file is created if desired.

9. BOXIM:

---------

(Largely obsolete, use AEDDSP unless special reasons for using BOXIM

e.g. Large map)

BOXIM cuts out a specified area from a standard format image file

It is usual first to TONE the file from which a circular

or polygonal area can be marked off.

A mean density is calculated from the edge points of the box.

For a circular area the edge coordinates are calculated line by line

to determine whether or not a point lies inside or outside the area.

For a polygonal area a line is constructed from each point vertically

upwards; if it crosses an even number of box sides it lies outside.

If not it lies inside and has the mean density subtracted from it

thus 'floating' the new image. (For points close to a box side the slope

of the line from the point to vertex 1 of the box side is calculated

and compared with the slope of the box side).

Finally the new image is moved to the bottom left hand corner of the new

box and the rest of the array is padded with zeroes.

Data input stream unit 5:

NXPAD, NYPAD, IPIXEL, NOVERT   (*)

OX, OY    (*)

For a circular box (NOVERT = 0) :

CX, CY, RAD    (*)

For a polygonal box (NOVERT > 0):

PX(I), PY(I)  (*)

NXPAD, NYPAD new padded box size e.g. 512 512

IPIXEL  pixel size of input file

NOVERT  0 for circular box.Number of vertices for polygonal box(up to 20)

OX, OY   coordinates in mm. of new phase origin.If set to 0.0 0.0 centre

of image assumed

CX, CY, RAD coordinates of centre of box and radius all measured in mm.

origin bottom left hand corner.

PX, PY X and Y coordinates of vertices which must go in circular(clockwise

or anticlockwise) order. THey are measured in mm. origin bottom left

hand corner.NOVERT of these records.

Input stream IN    densitometered input file

Output stream OUT    new boxed output file

EXAMPLE command file

$BOXIM IN PY42165.DAT OUT PY42165.OUT

$GO

256 256 1 6

40. 40.

40. 10.

20. 30.

20. 50.

40. 70.

60. 50.

60. 30.

10. BOXIMAGE:

------------

11. IMEDIT: Run online with prompts

----------

12. IMROTRAN :

------------

IMROTRAN is a program to rotate/ translate an image file (the relative file)

and calculate a correlation coefficient wrt another image file (the reference

file), then to output another image file of the average the reference file

with the best fit of the relative file.

INPUT DATA:

(40A1) Input image file (reference file does not move)

(40A1) Input image file (relative file is rotated/translated)

(40A1) Output image file (average of above two)

(*)    Theta1, Theta2, Thinc :

starting, finishing angles and increment in

degrees. A +ve angle rotates relative file

clockwise.

(*)    Xtrans1, Xtrans2, Xinc :

starting, finishing translation in X and increment

in pixels. A +ve value translates relative file

to the right.

(*)    Ytrans1, Ytrans2, Yinc :

starting, finishing translation in Y and increment

in pixels. A +ve value translates relative file

upwards.

(*)    Rmax :

maximum radius (in pixels) over which the correlation

coefficient is to be calculated.